CENTRAL ELECTRICITY REGULATORY COMMISSION

FORMULATION OF OPERATIONAL COST NORMS

FOR THERMAL GENERATION

REVIEW  PAPER

ON

BENCH MARKING INDUSTRY’S BEST PRACTICE

FOR

O & M COST AND ESCALATION CHARACTERISTIC OF SUCH COSTS

FOR

THERMAL POWER GENERATION

USING INTERNATIONAL AND DOMESTIC DATA

AUGUST 2000

DEVELOPMENT CONSULTANTS LIMITED

24B, PARK STREET, CALCUTTA  - 700 016

|  | # DOCUMENT CONTROL SHEET |
| --- | --- |

Project 	: FORMULATION OF OPERATIONAL COST NORMS

FOR THERMAL GENERATION

Client 	: CENTRAL ELECTRICITY REGULATORY COMMISSION

Document Title: REVIEW  PAPER ON BENCH MARKING INDUSTRY’S BEST PRACTICE FOR O & M COST AND ESCALATION CHARACTERISTIC OF SUCH COSTS FOR THERMAL POWER GENERATION USING INTERNATIONAL AND DOMESTIC DATA

Document No. 	: K0W02- RPT-O-0001

Rev. No. 	: 00

DCL Job No. 	: K0W02

ENDORSEMENTS

| 00 | 25.08.00 | Output1 | S.D. | D.K.S. | R.N.B. |
| --- | --- | --- | --- | --- | --- |
| REV. NO. | DATE | DESCRIPTION | PREP. BY SIGN(INITIAL) | REVW. BY SIGN(INITIAL) | APPRVD. BY SIGN(INITIAL) |

DEVELOPMENT CONSULTANTS LIMITED

CONSULTING ENGINEERS

CALCUTTA   MUMBAI  DELHI  CHENNAI

CENTRAL ELECTRICITY REGULATORY COMMISSION

FORMULATION OF OPERATIONAL COST NORMS

FOR THERMAL GENERATION

C O N T E N T S

1.0.0		PREAMBLE

2.0.0		APPROACH

3.0.0		OBSERVATION

3.1.0	US  PLANTS

3.2.0	FOREIGN   PLANTS

3.3.0	DOMESTIC PLANTS

4.0.0		TABLES

5.0.0		FIGURES

6.0.0		ATTACHMENTS

7.0.0		REFERENCES

REVIEW PAPER ON BENCH MARKING INDUSTRY’S BEST PRACTICE FOR O & M COST AND ESCALATION CHARACTERISTIC OF SUCH COSTS

FOR THERMAL GENERATION

1.0.0	PREAMBLE

From late seventies and early eighties there was a general practice among various thermal generation agencies that the base level Operation and Maintenance (O & M) expense for a thermal generation plant is to be 2.5 percent of the actual capital expenditure (CC) of the plant.  Further it was noted that among central sector generation agencies, the rule of thumb for escalation of O & M expense per annum was ten (10) percent of the base level O & M expense.

Accordingly, above stipulations were accepted and followed by all thermal generation agencies without raising any question till the business was subjected to competitive environment.  It was more so, since there was no incentive to improve the efficiency of operation.  However, with the advent of globalisation and liberalisation in the nineties, both central sector and state sector agencies have found that unless they become more and more competitive, it would be difficult for them to determine acceptable tariff for sale of electricity.

Against the above back drop, in the year 1997, Central Electricity Authority (CEA) laid down the following formula for determining annual   O & M cost of a thermal generation plant after the commercial operation date (COD) of the last unit.

C(O & M)n = 0.025 x CC (0.3 CPn/ CP1 + 0.7 WPn/WP1)

Where –

C(O & M)n	:	Annual O & M Expense  in the nth year of operation

CC	:	Actual Capital Expenditure

CPn	:	Consumer Price Index during the nth year of operation

CP1	:	Consumer Price Index during the first year of operation

WPn	:	Wholesale Price Index during the nth year of operation

WP1	:	Wholesale Price Index during the first year of operation

It is evident from above formula that the current tariff regulation is still guided by the base level O & M expense for thermal generation as 2.5% of the actual capital expenditure of the plant, with the exception of 4-stroke Diesel Engine Generating Station, for which base level O & M expense is 4%.

The current norm, however, is not acceptable to certain mega power projects, who intend to demand higher tariff than the existing rate.  Their demand is aimed to support the conception that the electricity generation cost has been appreciably affected by increasing stringency in health and environmental regulation.  The present study explores whether the above claim is justified with respect  to base level O & M expense experienced in both domestic and international best practices.

2.0.0	APPROACH

Present study is carried out with a view to ensure that different utilities do aspire to perform at a very high level of efficiency and availability.

In order to establish above performance, the procedure followed, for bench marking O & M cost norms, consists of reviewing international best practice as well as analysing existing indigenous O & M practice.

The study also explored to link O & M Cost with the Capital Cost as well as the Generation Cost.

It was never intended during this study to set apart fixed component of O & M cost from its variable component, since the contribution of latter component is not of much significance.

In view of above guidelines, following steps have been taken to proceed with the study.

1.	Interaction with Financial Times Energy, USA, and processing electronic data through internet pertaining to O & M of 1200 thermal generation plants, consisting of 8000 units, of USA.

2.	Scrutinized O & M practice of sixteen (16) countries as delineated in the document titled “ Projected Costs of Generating Electricity: Update 1998”.

3.	Interacted with various central sector and state sector domestic thermal generation agencies.

Data, thus collected, revealed that for similar technologies, the O & M cost in different countries varies within a wide range.  Reasons for such variation are explained elsewhere in this study.

3.0.0	OBSERVATIONS

3.1.0	US PLANTS

As in many other countries, the utilities in USA are changing their business practices to adapt to a competitive industry.

USA being a vast country, there is regional variations in material, labour and fuel costs across the country.  Notwithstanding above, levelised costs have been considered for carrying out the present study.

Out of 1200 US Plants, the present study scrutinised and processed various data restricted to as under:

	Steam Power (ST) Station:  Unit capacity between 200 MW and 500 MW and Plant load factor not less than 50%.

(Ref: ATTACHMENT 1)

	Combined Cycle Combustion Turbine (CCCT) Station:  Unit capacity between 100 MW and 350 MW and Plant load factor not less than 40%.

(Ref: ATTACHMENT 2)

	Diesel Engine Generating (DG) Station:  Unit capacity upto 50 MW and Plant load factor not less than 30%.

(Ref: ATTACHMENT 3)

	Non-Utility plants excluded.

Above restriction was necessary to study plants similar to those operating in India.  It may also be noted that for competitive economy abroad, many of the CCCT and DG stations are used as Peaking Units.  Eventually PLF of these plants is recorded to be low.

3.1.1	Plant Cost (Capital Cost)

In order to form a levelised Plant Cost  ($ / kW) along with its escalation over the years, following electronic data have been processed, which form the basis of present study.

	ST Station – Average Plant Cost of nine (9) plants, operating between 1988 and 1998 and whose last unit was installed on between 1955 and 1986, is summarised in TAB 1.  Plot of these data reveals a linear trend (Ref. ; Fig - 1).

	CCCT Station – TAB 2 shows Average Plant Cost of six (6) plants, operating between 1992 and 1998 and whose last unit was installed on between 1990 and 1996.  Plot of these data also reveals a linear trend (Ref.: Fig - 4).

	DG Station – As explained earlier DG units are used for peaking duty.  As a result, only two (2) operating plants could be traced on the Internet whose PLF is above 30%.  Such data, hence, have not been utilised to delineate the benchmark of the industry.

Average Plant Cost of these plants, operating between 1995 and 1998 and whose last unit was installed on 1991 and 1995, is presented in TAB 3.

3.1.2	Generation Cost

Electronic data, processed through Internet, revealed that as per current US practice, Generation Cost constitutes Fuel Cost, Non-Fuel Operating Cost and Maintenance Cost only.  Insurance is never used as a contributory factor to determine Generation Cost in USA.

Generation Cost of different type of plants is presented in TAB 1,2 & 3.

3.1.3	O & M Cost

Average O & M Cost along with its percentage with respect to Plant Cost and Generation Cost, for different plants, have been scrutinised and processed.  Results are summarised in tabular form and delineated in different figures.

	ST Station – O & M Cost, for those plants operating during the period 1988 to 1998, is presented in TAB 1 and Fig. 2 & 3.

	CCCT Station – TAB 2 and Fig. 5 & 6 show the O & M cost of different CCCT stations for the operating period 1992 to 1998.

	DG Station  – O & M Cost for DG Stations is reflected on TAB 3 and Fig. 7 for the operating period 1995 to 1998.

For the reference stations Average O & M Cost is 9.88% of the Plant Cost and 35.72% of the Generation Cost.

From Fig. 2, 5 & 7 it is evident that O & M expense of ST Station, CCCT Station and DG Station, with respect to Plant Cost, experiences a declining trend annually.

3.1.4	BENCH MARKING

Based on above, best operating practice followed in USA is as under:

ST		CCCT

O & M cost expressed

In terms of Capital Cost,	%	1.58		4.20

O & M cost expressed in

Terms of Generation Cost,   %	17.46		16.68

Annual escalation rate of O & M

Cost with respect to base year

1990-91,                               %	5.02		16.55

3.2.0	FOREIGN   PLANTS

Findings discussed in this section is based on the document “Projected Costs of Generating Electricity: Update 1998” prepared by Nuclear Energy Agency (NEA), International Energy Agency (IEA) and Organisation for Economic Co-operation and Development (OECD).

The main objective of the document is to explain information on estimated costs of generating electricity by base-load power plants and is focussed on technologies and plant types that could be commissioned in the respondent countries (total sixteen in number) on a common date, i.e., first of January, 2005.  The method adopted for calculating generation cost in this document is constant money levelised lifetime cost.  Constant money is expressed in respective national currency of each country as of first July, 1996 and is converted to US dollars of the same date for presenting summary table on a common monetary basis.

For estimating average levelised generation costs, the document has adopted a common economic lifetime of 40 years and a common plant load factor of 75 percent.  The document explains that the levelised cost methodology discounts the time series of expenditures and incomes to their present values in a specified year by applying a discount rate.  A discount rate takes into account, the time value of money.  In the present study the discount rate is used as 10 per cent per annum.

The costs reported in this document include plant specific cost components borne by thermal generation agencies including investment, operation and maintenance (O & M) and fuel costs as well as costs related to pollution control, waste management and other health and environmental protection measures.  However, cost elements that do not affect the relative competitiveness, e.g. central overheads, transmission and distribution costs, depreciation, etc., are excluded.

`	Based on above assumptions and guidelines, various expenses incurred in a thermal generation plant are summarised in TAB 4 & 5.

3.2.1	Base Construction Cost (Capital Cost)

From the table it could be visualized that the Base Construction Cost of Steam Power Station varies widely from 837 US $ / KW for 4 x 750 MW units in Canada to 2561 US $ / KW for 1 x 930 MW Unit in Japan.  Major factors, which lead to such wide variation in the cost among various countries, are labour cost, extent of automation adopted, domestic prices for equipment and material, statutory and regulatory stipulations, infrastructure, site specific conditions, etc.

3.2.2	Generation Cost

For the estimation of Generation Cost, it is emphasized, there is no unique and universal methodology. Hence, Projected Generation Cost, expressed in US Cent / KWh, is levelised with the assumption of 75% PLF and 40 year economic life time, which offers a sound basis for comparing relative international practice.  As in Construction Cost, Generation Cost of ST Generating Station also varies widely among different countries and ranges from 7.614 US Cent / KWh in Japan to 3.703 US Cent/KWh in Canada.  Factors, which contribute variations in Generation Cost, are cost of fuel, cost incurred for control of environmental emissions, cost of maintenance, wages & salaries of staff, etc.

3.2.3	O & M Cost

O & M Cost is reported in this document in US $ per KW capacity of a plant per year.  Items which constitute O & M cost is as under:

	Operation

	Maintenance (Man, Material & Service)

	Engineering Support Staff

	Administration

	General Expenses of Central Services

	Taxes & Duties (Plant Specific)

	Insurance (Plant Specific)

	Operating Waste Disposal (Ash, Sludge, etc.)

Insurance, however, is excluded in Russia and Spain.

The table reveals that the Projected O & M cost for Steam Power Station ranges from as low as 17.50 US $ / KW / Year in Brazil to as high as 81.33 US $ / KW / Year in Japan.

That the O & M cost varies widely among different countries is not unusual.  The variation is due to various factors, viz., wages & salaries of staff, regulations that affect O & M costs, productivity, number of personnel, etc.

3.2.4	BENCH MARKING

From TAB 4 & TAB 5 it could be easily visualized that industry’s best practice among different countries is experienced in Brazil for Steam Turbine Generating Station.

For CCCT, although best practice for O & M Cost per Capital Cost is experienced in Brazil, it is Turkey, which experienced best practice for O & M Cost per Generation Cost.  As regards Diesel Engine Generation Plant, it was not possible to procure any international data to aid the study. The document is also silent about escalation of O & M cost.  For ease of understanding, above findings are summarised as under:

ST		CCCT

	O & M cost expense in

Relation to Base Construction		1.39		1.13

Cost, %

	O & M cost expense in relation

To levelised generation cost		5.08		2.77

With 10% discount rate, %

3.3.0	DOMESTIC PLANTS

For analysing performance of domestic Steam Power (ST) Station, salient information, viz., detail O & M Cost Data, Project cost, Generation Cost, etc.  have been procured from various state sector and central sector thermal generation authorities.

As regards, CCCT Station, inputs were received from NTPC mainly with very little contribution from MSEB.

ATTACHMENT 4 constitutes information received for ST Station and CCCT Station.

There was however no respondent for Diesel Engine Generating (DG) Station.  Data furnished in this study is based on very restricted interaction with GMR VASAVI 4 x 50 MW Diesel Generation Power Project, Basin Bridge, Chennai.

3.3.1	Project (Capital) Cost

In order to cast forward rate of change of Project Cost (Rs./KW) for ST Station per annum, unit capacity is restricted between 200 MW and 500 MW, as is the trend for incoming installation. Associated data are collected for the period 1985-’86 to 1998 – ’99.  These data have been plotted and the relationship between “Project Cost and Year” is found to be linear in nature.  [Ref: TAB 6 and Fig 8.]   This trend forms the basis of levelised cost for analysis of various data in this study.

3.3.2	Generation Cost

As regards Generation cost, data available from various utilities for ST are too low to formulate a national norm.  The study is further aggravated by the approach taken by different utilities, which is very incoherent in nature.  In certain plants (Santaldih TPS, Kolaghat TPS) the cost of consumable is merged with the fuel cost.  Cost of spare is another item that is included with the O & M expense in one plant (Kolaghat TPS) and forms a part of fuel cost in another (North Madras).

In view of above this study recommends that a national guideline be formulated so as to identify the items, by their unique nature, which should form the Generation Cost of a plant.

Nevertheless, to circumvent the current aberration, published data of ST Station on cost structure of SEBs for the period 1992-93 to 1998-99 have been used to determine the Generation cost.  These are presented in ATTACHMENT 5 (eight sheets).

Information on Generation Cost for CCCT could not be procured from the utilities.

3.3.3	O & M Cost

O & M Cost for ST Station (in Rs. Lacs/MW) is summarised in TAB – 7, wherefrom O & M Cost / Project Cost (in percent) is calculated and presented in TAB – 8 and Fig. 9, which indicates that industry’s best practice is maintained at Kolaghat TPS during the last 6 years of operation.

Further, from sheet 8 of ATTACHAMENT 5, it is evident that industry’s best practice, for share of O & M Cost in Total Generation Cost, is maintained by Tamil Nadu Electricity Board at 2.66%.   However, plant specific limited data (Ref: TAB 10) reveal that it is Vijayawada Thermal Power Station that continues with industry’s best practice at 4.32%.

3.3.4	BENCH MARKING

Industry’s best practice to incur O & M expenditure is

ST		CCCT		DG

	O & M Cost in comparison

with Capital Cost,	%		1.97		3.80		2.50

	O & M Cost in comparison

with Generation cost,  %		2.66		 -		6.50

	Annual escalation

of O & M cost with respect to

base year 1990-91,       %		14.04		-		7.1

4.0.0	TABLES

TAB 1	:	PLANT COST,  O & M  COST  AND GENERATION COST, ST  STATION, USA

TAB 2	:	PLANT COST,  O & M  COST  AND GENERATION COST, CCCT STATION, USA

TAB 3	:	PLANT COST,  O & M  COST  AND GENERATION COST, DG STATION, USA

TAB 4	:	DATA BASE,  ST STATION, FOREIGN COUNTRIES

TAB 5	:	DATA BASE, CCCT  STATION,  FOREIGN COUNTRIES

TAB 6	:	PROJECT COST,  ST STATION, DOMESTIC

TAB 7	:	O & M  COST, ST STATION, DOMESTIC

TAB 8	:	O & M COST/ PROJECT COST, ST STATION, DOMESTIC

TAB 9	:	O & M COST / GENERATION COST, ST STATION, DOMESTIC

TAB 10	:	O & M COST, CCCT  STATION, DOMESTIC

| TAB -1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Plant Cost , O & M Cost and Generation Cost, Steam Power Station,U.S.A. |  |  |  |  |  |  |
| Year | Plant Cost, $/KW | Total Non-Fuel O&M Cost $/KW | Total Non-Fuel O&M Cost $/MWh | Total Generation Cost $/MWh | O&M Cost / Plant Cost % | O&M Cost/Generation Cost % |
| 1988 | 818.94 | 16.03 | 3.32 | 19.07 | 1.96 | 16.43 |
| 1989 | 864.61 | 14.94 | 3.04 | 18.98 | 1.73 | 15.35 |
| 1990 | 910.28 | 13.23 | 3.12 | 18.99 | 1.45 | 15.56 |
| 1991 | 955.95 | 14.29 | 3.09 | 18.36 | 1.50 | 15.86 |
| 1992 | 1001.62 | 15.10 | 3.31 | 18.80 | 1.51 | 16.73 |
| 1993 | 1047.30 | 16.02 | 2.93 | 17.40 | 1.53 | 16.57 |
| 1994 | 1092.97 | 17.27 | 3.22 | 18.21 | 1.58 | 17.61 |
| 1995 | 1138.64 | 17.91 | 3.65 | 18.30 | 1.57 | 19.55 |
| 1996 | 1184.31 | 18.05 | 3.45 | 17.97 | 1.52 | 18.45 |
| 1997 | 1229.98 | 18.14 | 3.48 | 17.63 | 1.47 | 19.19 |
| 1998 | 1275.66 | 19.53 | 3.72 | 17.31 | 1.53 | 20.72 |
| Average | 1047.30 | 16.41 | 3.30 | 18.27 | 1.58 | 17.46 |

| TAB -2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Plant Cost , O & M Cost and Generation Cost, CCCT Station,U.S.A. |  |  |  |  |  |  |
| Year | Plant Cost $/KW | Total Non-Fuel O&M Cost, $/KW | Total Non-Fuel O& M Cost , $/MWh | Total Generation Cost , $/MWh | O&M Cost /Plant Cost, % | O&M Cost/Generation Cost, % |
| 1992 | 606.64 | 22.84 | 4.35 | 25.52 | 3.76 | 18.05 |
| 1993 | 494.38 | 26.95 | 5.91 | 36.28 | 5.45 | 14.53 |
| 1994 | 576.18 | 28.64 | 5.53 | 33.59 | 4.97 | 16.34 |
| 1995 | 657.98 | 31.34 | 6.53 | 37.26 | 4.76 | 17.20 |
| 1996 | 739.79 | 26.71 | 6.33 | 37.01 | 3.61 | 17.42 |
| 1997 | 821.59 | 29.88 | 6.73 | 39.07 | 3.64 | 17.83 |
| 1998 | 903.39 | 29.04 | 5.91 | 35.58 | 3.21 | 15.37 |
| Average | 685.71 | 27.91 | 5.90 | 34.90 | 4.20 | 16.68 |

| TAB -3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Plant Cost , O & M Cost and Generation Cost, Diesel Engine Generating Station,U.S.A. |  |  |  |  |  |  |  |
| Year | Plant Cost, $/KW | Total Non-Fuel O&M Cost, $/KW | Total Non-Fuel O&M Cost, $/MWh | Generation Cost ,$ | Generation Cost, $/MWh | O&M Cost/Plant Cost , % | O&M Cost/Generation Cost , % |
| 1995 | 731.71 | 85.05382151 | 17.91 | 14699973 | 70.82 |  |  |
| 1995 | 1464.63 | 132.7878723 | 49.33 | 2975238 | 117.59 |  |  |
| 1995 | 1098.17 | 108.92 | 33.62 | 8837605.5 | 94.205 | 9.92 | 35.69 |
| 1996 | 1202.23 | 83.93 | 18.30 | 16135280 | 80.5 |  |  |
| 1996 | 1522.17 | 201.69 | 71.03 | 4027823 | 150.91 |  |  |
| 1996 | 1362.2 | 142.81 | 44.67 | 10081551.5 | 115.705 | 10.48 | 38.60 |
| 1997 | 1326.51 | 91.05 | 19.55 | 17233121 | 84.67 |  |  |
| 1997 | 1522.63 | 156.84 | 54.04 | 3677769 | 134.81 |  |  |
| 1997 | 1424.57 | 123.94 | 36.80 | 10455445 | 109.74 | 8.70 | 33.53 |
| 1998 | 731.71 | 72.52 | 13.57 | 18213606 | 78 |  |  |
| 1998 | 1522.63 | 162.03 | 54.31 | 3239478 | 115.51 |  |  |
| 1998 | 1127.17 | 117.28 | 33.94 | 10726542 | 96.755 | 10.40 | 35.08 |
| Average | 1284.00 | 126.10 | 37.98 | 10262822.10 | 106.08 | 9.88 | 35.72 |

|  | TAB - 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Database , ST Station, Foreign Countries Except U.S.A. |  |  |  |  |  |  |  |  |  |
|  | [ REF : PROJECTED COST OF GENERATING ELECTRICITY UPDATE - 1998 ] |  |  |  |  |  |  |  |  |  |
| Sl. No. |  | Country | Capacity MW | Emission Control Equipment | Base Construction Cost, US$ of 01.07.96 /KW |  |  |  |  |  |
|  |  |  |  |  |  | ## Projected O & M Cost in 2005 |  | Projected O & M Cost & Generation Cost, at 10% Discount Rate |  |  |
|  |  |  |  |  |  |  |  | US$/1000 of 01.07.96/kWh |  |  |
|  |  |  |  |  |  | US$ of 01.07.96/KW /Year | % of Base Construction Cost | O & M | O & M /Total % | Total |
| 1 |  | Belgium | 1x400 | FGD,DeNOx,ESP | 1386 | 66.8 | 4.82 | 10.4 | 19.82 | 52.47 |
| 2 |  | Canada | 4x750 | FGD,SCR | 837 | 26.01 | 3.11 | 4.05 | 10.94 | 37.03 |
|  |  |  | 4x200 | FGD | 1360 | 50.55 | 3.72 | 7.87 | 14.52 | 54.19 |
| 3 |  | Denmark | 1x400 | FGD,DeNOx,ESP | 1329 | 43.87 | 3.30 | 6.83 | 13.97 | 48.9 |
| 4 |  | Finland | 1x500 | FGD,SCR | 885 | 57.71 | 6.52 | 8.99 | 22.99 | 39.11 |
| 5 |  | France | 1x572 | FGD,SCR,ESP | 1346 | 64.43 | 4.79 | 10.03 | 16.85 | 59.54 |
| 6 |  | Hungary | 1x120 |  | 1227 | 58.35 | 4.76 | 9.09 | 19.21 | 47.33 |
|  |  |  | 2x459 | FGD | 1318 | 37.73 | 2.86 | 5.88 | 12.89 | 45.62 |
| 7 |  | Italy | 4x617 | FGD,SCR,ESP | 1173 | 39.75 | 3.39 | 6.19 | 11.74 | 52.73 |
| 8 |  | Japan | 1x930 | FGD,DeNOx,Dust | 2561 | 81.33 | 3.18 | 12.66 | 16.63 | 76.14 |
| 9 |  | Korea | 1x500 | FGD,SCR | 1174 | 53.13 | 4.53 | 8.27 | 18.39 | 44.96 |
| 10 |  | Netherland | 1x600 | FGD,SCR | 1254 | 53.35 | 4.25 | 8.31 | 14.71 | 56.48 |
|  |  |  | 1x600 | FGD,SCR | 1450 | 53.35 | 3.68 | 8.31 | 14.01 | 59.33 |
|  |  |  | 1x800 | FGD,DeNOx | 1553 | 54.51 | 3.51 | 8.49 | 13.79 | 61.55 |
| 11 |  | Portugal | 1x315 | FGD,SCR,ESP | 1999 | 74.76 | 3.74 | 11.64 | 16.76 | 69.44 |
|  |  |  | 1x411 | FGD,SCR,ESP | 1902 | 73.85 | 3.88 | 11.5 | 17.26 | 66.62 |
| 12 |  | Spain | 1x500 | FGD,ESP,LNB | 1326 | 39.72 | 3.00 | 6.18 | 11.30 | 54.67 |
| 13 |  | Turkey | 1x500 | FGD,DeNOx,Dust | 1019 | 58.13 | 5.70 | 9.05 | 18.58 | 48.7 |
|  |  |  | 4x85 | FGD,DeNOx,Dust | 1476 | 49.18 | 3.33 | 7.66 | 10.98 | 69.74 |
| 14 |  | USA | 1x300 | FGD,ESP,LNB | 1009 | 33.5 | 3.32 | 5.22 | 15.04 | 34.71 |
|  |  |  | 1x380 | LNB,ESP | 1154 | 32.84 | 2.85 | 5.11 | 14.31 | 35.7 |
| 14 |  | Brazil | 1x315 | ESP | 1258 | 17.5 | 1.39 | 3.12 | 7.22 | 43.2 |
|  |  |  | 1x315 | ESP | 1094 | 17.69 | 1.62 | 3.14 | 5.08 | 61.8 |
| 15 |  | China | 2x600 | FGD,ESP | 772 | 36.05 | 4.67 | 5.61 | 14.04 | 39.96 |
| 17 |  | India | 2x460 | ESP | 935 | 27.89 | 2.98 | 4.34 | 10.80 | 40.17 |
| 16 |  | Russia | 4x300 | FGD | 1291 | 32.99 | 2.56 | 7.35 | 13.28 | 55.34 |

| TAB - 5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Database , CCCT Station, Foreign Countries Except U.S.A. |  |  |  |  |  |  |  |  |  |  |
| [ REF : PROJECTED COST OF GENERATING ELECTRICITY UPDATE - 1998 ] |  |  |  |  |  |  |  |  |  |  |
| Sl. No. | Country | Capacity MW | Emission Control Equipment | Base Construction Cost US$ of 01.07.96 /KW | Projected O & M Cost in 2005 |  | Projected O & M Cost & Generation Cost, at 10% Discount Rate |  |  |  |
|  |  |  |  |  |  |  | US$/1000 of 01.07.96/kWh |  |  |  |
|  |  |  |  |  | US$ of 01.07.96/KW /Year | % of Base Construction Cost | O & M | O & M /Total % | Total |  |
| 1 | Belgium | 1x350 | LNB | 761 | 45.32 | 5.96 | 7.06 | 16.68 | 42.33 |  |
| 2 | Canada | 2x750 |  | 536 | 13.04 | 2.43 | 2.03 | 6.14 | 33.04 |  |
| 3 | Denmark | 1x337 | DeNOx | 809 | 27.8 | 3.44 | 4.33 | 8.34 | 51.94 |  |
|  |  | 1x400 | DeNOx | 885 | 67.05 | 7.58 | 5.77 | 9.99 | 57.77 |  |
| 4 | Finland | 2x350 |  | 622 | 70.63 | 11.36 | 11 | 26.78 | 41.07 |  |
| 5 | France | 1x660 | LNB | 739 | 28.53 | 3.86 | 4.44 | 8.32 | 53.35 |  |
| 6 | Hungary | 1x389 |  | 595 | 21.17 | 3.56 | 3.49 | 8.65 | 40.37 |  |
| 7 | Italy | 2x350 | LNB | 652 | 16.94 | 2.60 | 2.64 | 5.14 | 51.32 |  |
| 8 | Japan | 2x343 | DeNOx | 1640 | 51.11 | 3.12 | 7.96 | 9.43 | 84.4 |  |
| 9 | Korea | 1x450 | LNB | 583 | 21.8 | 3.74 | 3.39 | 7.22 | 46.98 |  |
| 10 | Netherland | 1x250 |  | 725 | 24.59 | 3.39 | 3.83 | 7.93 | 48.31 |  |
|  |  | 1x350 |  | 664 | 23.71 | 3.57 | 3.69 | 8.39 | 43.99 |  |
| 11 | Portugal | 1x326 | LNB | 790 | 16.1 | 2.04 | 2.51 | 5.04 | 49.79 |  |
|  |  | 1x459 | LNB | 697 | 15.04 | 2.16 | 2.34 | 4.83 | 48.49 |  |
| 12 | Spain | 1x315 | LNB | 663 | 31.37 | 4.73 | 4.88 | 8.98 | 54.36 |  |
| 13 | Turkey | 2x340 | DeNOx | 402 | 6.07 | 1.51 | 0.94 | 2.77 | 33.94 |  |
| 14 | USA | 1x250 | SCR | 422 | 18.09 | 4.29 | 2.82 | 10.30 | 27.37 |  |
|  |  | 1x350 | SCR | 419 | 17.42 | 4.16 | 2.71 | 11.48 | 23.6 |  |
| 14 | Brazil | 2x450 |  | 677 | 8.39 | 1.24 | 1.45 | 4.43 | 32.73 |  |
|  |  | 1x450 |  | 766 | 8.62 | 1.13 | 1.47 | 4.22 | 34.87 |  |
| 15 | Russia | 4x360 |  | 721 | 20.09 | 2.79 | 4.05 | 10.39 | 38.99 |  |

| Table - 6 |  |  |
| --- | --- | --- |
|  | Project Cost, ST Station, Domestic |  |
| Year | Name of the Project & Capacity (No x MW) | Project Cost , Rs./KW |
| 85-86 | Ukai - III, 1 x 210 | 5099 |
| 86-87 | - |  |
| 87-88 | Wanakbori - II, 3 x 210 | 6816 |
| 87-88 | Neyveli TPS Stg - I, 3 x 210 | 8258 |
| 88-89 | Singrauli STPS, 5 x 200 + 2 x 500 | 5954 |
| 89-90 | Vijayawada - II, 2 x 210 | 12175 |
| 90-91 | Gandhinagar - II, 2 x 210 | 12945 |
| 90-91 | Rihand STPS, 2 x 500 | 23900 |
| 91-92 | Neyveli TPS Stg - II, 4 x 210 | 17208 |
| 92-93 | - |  |
| 93-94 | - |  |
| 94-95 | Vijayawada - III, 2 x 210 | 20000 |
| 94-95 | Rayalaseema – I, 2 x 210 | 20483 |
| 94-95 | NCTPP Dadri, 4 x 210 | 19872 |
| 95-96 | IB Valley - I, 2 x 210 | 23174 |
| 95-96 | Farakka STPS, 3 x 200 + 2 x 500 | 19901 |
| 95-96 | Kolaghat TPS - II, 3 x 210 | 17778 |
| 95-96 | North Madras, 3 x 210 | 22168 |
| 96-97 | - |  |
| 97-98 | - |  |
| 98-99 | Kothagudem TPS Stg - V, 2 x 250 | 28480 |

| TAB - 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O & M COST, Rs. in Lacs / MW, ST Station, Domestic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant | 92-93 | 93-94 |  |  | 94-95 |  | 95-96 |  | 96-97 |  | 97-98 |  | 98-99 |  | 99-2K |  |  |
| Singrauli STPS | 3.13 | 4.02 |  |  | 5.23 |  | 5.17 |  | 5.38 |  | 8.09 |  | 8.08 |  | 10.63 |  |  |
| Rihand STPS | 4.07 | 4.29 |  |  | 5.24 |  | 5.67 |  | 7.51 |  | 8.72 |  | 9.06 |  | 8.56 |  |  |
| Farakka STPS | 8.89 | 11.09 |  |  | 12.48 |  | 7.35 |  | 9.82 |  | 8.36 |  | 8.36 |  | 9.64 |  |  |
| Unchahar -I TPS | 5.39 | 7.23 |  |  | 9.25 |  | 8.76 |  | 11.25 |  | 10.78 |  | 13.66 |  | 16.42 |  |  |
| NCTPP Dadri TPS | 7.70 | 6.25 |  |  | 7.35 |  | 6.08 |  | 7.24 |  | 8.04 |  | 9.79 |  | 10.83 |  |  |
| Santaldih TPS |  |  |  |  | 4.49 |  | 4.78 |  | 5.09 |  | 6.64 |  | 7.74 |  | 9.45 |  |  |
| Kolaghat TPS |  |  |  |  | 3.46 |  | 4.31 |  | 4.06 |  | 4.85 |  | 5.65 |  | 7.62 |  |  |
| Neyveli TPS -I |  |  |  |  |  |  | 8.39 |  | 8.04 |  | 9.27 |  | 9.12 |  | 7.52 |  |  |
| Neyveli TPS -II , Stage I |  |  |  |  |  |  | 6.47 |  | 6.07 |  | 7.01 |  | 7.34 |  | 7.10 |  |  |
| Neyveli TPS -II , Stage II |  |  |  |  |  |  | 6.55 |  | 6.11 |  | 7.02 |  | 7.66 |  | 7.30 |  |  |
| North Madras TPS |  |  |  |  |  |  |  |  |  |  | 7.84 |  | 11.03 |  | 9.54 |  |  |
| TAB - 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| O & M COST / PROJECT COST, %, ST Station , Domestic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92-93 | 93-94 |  |  | 94-95 |  | 95-96 |  | 96-97 |  | 97-98 |  | 98-99 |  | 99-2K |  | Average |
| Project Cost , Rs./KW | 16718.7 | 18551.6 |  |  | 20384.5 |  | 22217.4 |  | 24050.3 |  | 25883.2 |  | 27716.1 |  | 29549 |  |  |
| Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Singrauli STPS | 1.87 | 2.16 |  |  | 2.57 |  | 2.33 |  | 2.24 |  | 3.13 |  | 2.91 |  | 3.60 |  | 2.60 |
| Rihand STPS | 2.43 | 2.31 |  |  | 2.57 |  | 2.55 |  | 3.12 |  | 3.37 |  | 3.27 |  | 2.90 |  | 2.82 |
| Farakka STPS | 5.32 | 5.98 |  |  | 6.12 |  | 3.31 |  | 4.08 |  | 3.23 |  | 3.02 |  | 3.26 |  | 4.29 |
| Unchahar -I TPS | 3.22 | 3.90 |  |  | 4.54 |  | 3.94 |  | 4.68 |  | 4.17 |  | 4.93 |  | 5.56 |  | 4.37 |
| NCTPP Dadri TPS | 4.60 | 3.37 |  |  | 3.61 |  | 2.74 |  | 3.01 |  | 3.10 |  | 3.53 |  | 3.67 |  | 3.45 |
| Santaldih TPS |  |  |  |  | 2.20 |  | 2.15 |  | 2.12 |  | 2.57 |  | 2.79 |  | 3.20 |  | 2.50 |
| Kolaghat TPS |  |  |  |  | 1.70 |  | 1.94 |  | 1.69 |  | 1.88 |  | 2.04 |  | 2.58 |  | 1.97 |
| Neyveli TPS -I |  |  |  |  |  |  | 3.77 |  | 3.35 |  | 3.58 |  | 3.29 |  | 2.55 |  | 3.31 |
| Neyveli TPS -II , Stage I |  |  |  |  |  |  | 2.91 |  | 2.52 |  | 2.71 |  | 2.65 |  | 2.40 |  | 2.64 |
| Neyveli TPS -II , Stage II |  |  |  |  |  |  | 2.95 |  | 2.54 |  | 2.71 |  | 2.77 |  | 2.47 |  | 2.69 |
| North Madras TPS |  |  |  |  |  |  |  |  |  |  | 3.03 |  | 3.98 |  | 3.23 |  | 3.41 |
| Average | 3.49 | 3.54 |  |  | 3.33 |  | 2.86 |  | 2.93 |  | 3.04 |  | 3.20 |  | 3.22 |  |  |
| TAB - 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| O & M COST / GENERATION COST, %, ST Station, Domestic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant | 92-93 |  | 93-94 | 94-95 |  | 95-96 |  | 96-97 |  | 97-98 |  | 98-99 |  | 99-2K |  |  | Average |
| Singrauli STPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rihand STPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Farakka STPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unchahar -I TPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NCTPP Dadri TPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vijayawada TPS |  |  |  | 4.39 |  | 2.31 |  | 4.76 |  | 4.95 |  | 5.21 |  |  |  |  | 4.32 |
| Santaldih TPS |  |  |  | 17.6 |  | 15.77 |  | 16.32 |  | 16.53 |  | 18.63 |  |  |  |  | 16.97 |
| Kolaghat TPS |  |  |  | 6.13 |  | 6.2 |  | 6.45 |  | 6.59 |  | 7.04 |  |  |  |  | 6.48 |
| Neyveli TPS -I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Neyveli TPS -II , Stage I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Neyveli TPS -II , Stage II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| North Madras TPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average |  |  |  | 9.37 |  | 8.09 |  | 9.18 |  | 9.36 |  | 10.29 |  |  |  |  |  |
| TAB - 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| O & M COST , CCCT Station , Domestic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant | Project Cost, Rs./KW | O&M Cost |  |  | 93-94 |  | 94-95 |  | 95-96 |  | 96-97 |  | 97-98 |  | 98-99 | 99-2K | Average |
| Auraiya GPS | 10232 | Rs. Lacs/MW |  |  | 4.39 |  | 4.34 |  | 2.34 |  | 7.32 |  | 5.29 |  | 11.85 | 5.97 | 5.93 |
|  |  | % of Project Cost |  |  | 4.29 |  | 4.24 |  | 2.29 |  | 7.15 |  | 5.17 |  | 11.58 | 5.83 | 5.79 |
| Kawas GPS | 24376 | Rs. Lacs/MW |  |  | 3.92 |  | 4.28 |  | 4.02 |  | 10.76 |  | 8.40 |  | 11.87 | 21.62 | 9.27 |
|  |  | % of Project Cost |  |  | 1.61 |  | 1.76 |  | 1.65 |  | 4.41 |  | 3.45 |  | 4.87 | 8.87 | 3.80 |

5.0.0	FIGURES

Fig.	1	:	PLANT COST, ST STATION, USA

Fig.	2	:	O & M COST / PLANT COST, ST STATION, USA

Fig. 	3	:	O & M COST / GENERATION COST, ST STATION, USA

Fig.	4	:	PLANT COST, CCCT STATION, USA

Fig. 	5	:	O & M COST / PLANT COST, CCCT STATION, USA

Fig.	6	:	O & M COST / GENERATION COST, CCCT STATION, USA

Fig. 	7	:	O & M COST / PLANT COST AND O & M COST / GENERATION COST, DG STATION,  USA

Fig.	8	:	PROJECT COST, ST STATION, DOMESTIC

Fig.	9	:	O & M COST / PROJECT COST, ST STATION, DOMESTIC

Fig.	10	:	O & M COST / GENERATION COST, ST STATION, DOMESTIC

|  |  |  |  |  |  |  |  | 100 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 3.22 |  |  |

6.0.0	ATTACHMENTS

ATTACHMENT  	1	:	DATABASE, ST STATION,  USA

(3 SHEETS)

ATTACHMENT	2	:	DATABASE, CCCT STATION, USA

(1 SHEET)

ATTACHMENT	3	:	DATABASE, DG STATION, USA

(1 SHEET)

ATTACHMENT	4	:	DATABASE, ST STATION AND CCCT

(25 SHEETS)			STATION, DOMESTIC

ATTACHMENT	5	:	(GENERATION) COST STRUCTURE OF

(8 SHEETS)			SEBS, ST STATION, DOMESTIC

| Plant Name | Year | Year Last Unit Installed | Name Plate Capacity MW | Net Generation MWh | Cap Factor % | Total Cost of Plant $ | Fuel $ | Total Non-Fuel O&M $ | Total Production Costs $ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| McWilliams | 1988 | 1959 | 40 | 5510 | 1.48 | 0 | 191808 | 649952 | 841760 |
| McWilliams | 1989 | 1959 | 40 | 3924 | 1.06 | 0 | 151106 | 537770 | 688876 |
| McWilliams | 1990 | 1959 | 40 | 31055 | 8.37 | 0 | 935135 | 598435 | 1533570 |
| McWilliams | 1991 | 1959 | 40 | 12132 | 3.27 | 0 | 372856 | 599523 | 972379 |
| McWilliams | 1992 | 1959 | 40 | 12162 | 3.27 | 0 | 394775 | 697169 | 1091944 |
| McWilliams | 1993 | 1959 | 40 | 13854 | 3.74 | 0 | 564562 | 759529 | 1324091 |
| McWilliams | 1994 | 1959 | 40 | 9688 | 2.61 | 0 | 392515 | 929670 | 1322185 |
| McWilliams | 1995 | 1959 | 40 | 8334 | 2.25 | 0 | 369556 | 662716 | 1032272 |
| Port Allen | 1995 | 1969 | 10 | 57092 | 65.17 | 5641816 | 0 | 1078931 | 1078931 |
| Port Allen | 1996 | 1969 | 10 | 53525 | 60.93 | 9876201 | 0 | 1106224 | 1106224 |
| Port Allen | 1997 | 1969 | 10 | 54180 | 61.85 | 9950758 | 0 | 1231420 | 1231420 |
| Port Allen | 1998 | 1969 | 10 | 48833 | 55.75 | 5641816 | 0 | 1197428 | 1197428 |
| Haynes Generating Station | 1988 | 1967 | 1606 | 4075286 | 29.55 | 235514398 | 87881289 | 18211531 | 106092820 |
| Haynes Generating Station | 1989 | 1967 | 1606 | 3602055 | 26.19 | 234497100 | 39016837 | 29758959 | 68775796 |
| Haynes Generating Station | 1990 | 1967 | 1606 | 3736635 | 27.17 | 253376908 | 107013876 | 25104984 | 132118860 |
| Haynes Generating Station | 1991 | 1967 | 1606 | 2485700 | 18.07 | 285156148 | 84022438 | 27442416 | 111464854 |
| Haynes Generating Station | 1992 | 1967 | 1606 | 2953910 | 21.42 | 291816106 | 90933939 | 26657594 | 117591533 |
| Haynes Generating Station | 1993 | 1967 | 1606 | 3821331 | 27.79 | 312423722 | 137266379 | 26860732 | 164127111 |
| Haynes Generating Station | 1994 | 1967 | 1606 | 3050234 | 22.18 | 333054888 | 107109941 | 29994567 | 137104508 |
| Haynes Generating Station | 1995 | 1967 | 1606 | 3194981 | 23.23 | 338725590 | 94357682 | 22506933 | 116864615 |
| Haynes Generating Station | 1996 | 1967 | 1606 | 1326671 | 9.62 | 339183998 | 36649154 | 22085396 | 58734550 |
| Haynes Generating Station | 1997 | 1967 | 1606 | 719618 | 5.23 | 343645778 | 36804207 | 31071000 | 67875207 |
| Haynes Generating Station | 1998 | 1967 | 1606 | 554379 | 4.03 | 344772910 | 32797824 | 29610176 | 62408000 |
| Linden | 1988 | 1972 | 612.94 | 1185765 | 27.05 | 161336939 | 30240031 | 17321171 | 47561202 |
| Linden | 1989 | 1972 | 612.94 | 1302053 | 29.79 | 162946968 | 37833576 | 17112598 | 54946174 |
| Linden | 1990 | 1972 | 612.94 | 544699 | 12.46 | 168077300 | 18456622 | 14429507 | 32886129 |
| Linden | 1991 | 1972 | 612.94 | 653975 | 14.81 | 171237167 | 21228238 | 16775808 | 38004046 |
| Linden | 1992 | 1972 | 612.94 | 486033 | 10.98 | 172593728 | 16702265 | 13458688 | 30160953 |
| Linden | 1993 | 1972 | 612.94 | 70766 | 1.6 | 174151695 | 5331185 | 7772701 | 13103886 |
| Linden | 1994 | 1972 | 612.94 | 176649 | 4 | 141872692 | 9261390 | 7362665 | 16624055 |
| Linden | 1995 | 1972 | 612.94 | 116880 | 3.1 | 145674229 | 7073918 | 6475801 | 13549719 |
| Linden | 1996 | 1972 | 612.94 | 8217 | 0.22 | 145831909 | 0 | 0 | 0 |
| Linden | 1997 | 1972 | 519.44 | 18234 | 0.48 | 146753316 | 1632380 | 2175871 | 3808251 |
| Linden | 1998 | 1972 | 519.44 | 49401 | 1.31 | 146753316 | 2012960 | 1609821 | 3622781 |
| Colbert | 1988 | 1965 | 1350 | 8367982 | 76.52 | 309249037 | 153855132 | 18581852 | 172436984 |
| Colbert | 1989 | 1965 | 1350 | 6980489 | 66.18 | 314035298 | 135804256 | 18273234 | 154077490 |
| Colbert | 1990 | 1965 | 1350 | 5153786 | 48.86 | 312211766 | 81503388 | 21104178 | 102607566 |
| Colbert | 1991 | 1965 | 1350 | 6678833 | 63.32 | 318235718 | 97035576 | 19362454 | 116398030 |
| Colbert | 1992 | 1965 | 1350 | 5698990 | 53.89 | 320043519 | 87535853 | 23151300 | 110687153 |
| Colbert | 1993 | 1965 | 1350 | 7333973 | 69.54 | 419569491 | 91874560 | 19000120 | 110874680 |
| Colbert | 1994 | 1965 | 1350 | 6811462 | 64.58 | 514030485 | 100926401 | 25844446 | 126770847 |
| Colbert | 1995 | 1965 | 1350 | 6598102 | 62.56 | 607726219 | 89805935 | 24114618 | 113920553 |
| Colbert | 1996 | 1965 | 1350 | 7836469 | 74.1 | 631754833 | 102890579 | 25452633 | 128343212 |
| Colbert | 1997 | 1965 | 1350 | 6739698 | 63.9 | 621707561 | 86501463 | 27740085 | 114241548 |
| Colbert | 1998 | 1965 | 1350 | 6753512 | 64.03 | 650439343 | 89019248 | 29398375 | 118417623 |
| Johnsonville (TVA) | 1988 | 1959 | 1485.2 | 5736405 | 54.88 | 313586778 | 78200064 | 21002494 | 99202558 |
| Johnsonville (TVA) | 1989 | 1959 | 1485.2 | 4684950 | 42.65 | 318533230 | 65901493 | 19581020 | 85482513 |
| Johnsonville (TVA) | 1990 | 1959 | 1485.2 | 5002721 | 45.54 | 346994080 | 75317126 | 19799711 | 95116837 |
| Johnsonville (TVA) | 1991 | 1959 | 1485.2 | 4787658 | 43.58 | 352369707 | 69550102 | 20634313 | 90184415 |
| Johnsonville (TVA) | 1992 | 1959 | 1485.2 | 5010988 | 45.49 | 353250747 | 77739654 | 25191519 | 102931173 |
| Johnsonville (TVA) | 1993 | 1959 | 1485.2 | 7443489 | 67.76 | 354977159 | 110570914 | 25174390 | 135745304 |
| Johnsonville (TVA) | 1994 | 1959 | 1485.2 | 7690913 | 70.01 | 410767617 | 115217928 | 28513301 | 143731229 |
| Johnsonville (TVA) | 1995 | 1959 | 1485.2 | 6528917 | 59.43 | 475526517 | 88468176 | 33012093 | 121480269 |
| Johnsonville (TVA) | 1996 | 1959 | 1485.2 | 7486226 | 67.96 | 572305513 | 102092825 | 35036285 | 137129110 |
| Johnsonville (TVA) | 1997 | 1959 | 1485.2 | 6644844 | 60.49 | 586868537 | 90594321 | 29760401 | 120354722 |
| Johnsonville (TVA) | 1998 | 1959 | 1485.2 | 6965680 | 63.41 | 600642302 | 90141116 | 38821213 | 128962329 |
| Kingston | 1988 | 1955 | 1700 | 9499491 | 74.69 | 316332987 | 119173848 | 25659523 | 144833371 |
| Kingston | 1989 | 1955 | 1700 | 9014076 | 70.67 | 320527950 | 109538666 | 21723687 | 131262353 |
| Kingston | 1990 | 1955 | 1700 | 8670360 | 67.98 | 338586415 | 114897083 | 21312558 | 136209641 |
| Kingston | 1991 | 1955 | 1700 | 9637799 | 75.56 | 342614150 | 123030545 | 25858651 | 148889196 |
| Kingston | 1992 | 1955 | 1700 | 8994754 | 70.33 | 358139324 | 115852614 | 24463913 | 140316527 |
| Kingston | 1993 | 1955 | 1700 | 10129502 | 79.42 | 383362630 | 123629768 | 25050737 | 148680505 |
| Kingston | 1994 | 1955 | 1700 | 9980963 | 78.25 | 391172558 | 127499171 | 29806463 | 157305634 |
| Kingston | 1995 | 1955 | 1700 | 10192073 | 79.91 | 402208789 | 128563597 | 29467454 | 158031051 |
| Kingston | 1996 | 1955 | 1700 | 9858167 | 77.08 | 444120708 | 123763687 | 35337704 | 159101391 |
| Kingston | 1997 | 1955 | 1700 | 9326416 | 73.12 | 473630817 | 116400501 | 32618771 | 149019272 |
| Kingston | 1998 | 1955 | 1700 | 10264053 | 80.47 | 482793250 | 127594901 | 33353748 | 160948649 |
| Plant Name | Year | Year Last Unit Installed | Name Plate Capacity MW | Net Generation MWh | Cap Factor % | Total Cost of Plant $ | Fuel $ | Total Non-Fuel O&M $ | Total Production Costs $ |
| Widows Creek | 1988 | 1965 | 1968.76 | 4748864 | 31.4 | 479925148 | 72652063 | 34344326 | 106996389 |
| Widows Creek | 1989 | 1965 | 1968.76 | 5902341 | 41.77 | 504504584 | 82358249 | 32308009 | 114666258 |
| Widows Creek | 1990 | 1965 | 1968.76 | 7043362 | 49.85 | 533483651 | 94866068 | 36393202 | 131259270 |
| Widows Creek | 1991 | 1965 | 1968.76 | 6138477 | 43.44 | 547037304 | 92677612 | 35287760 | 127965372 |
| Widows Creek | 1992 | 1965 | 1968.76 | 7471701 | 52.73 | 599045567 | 100325817 | 37910819 | 138236636 |
| Widows Creek | 1993 | 1965 | 1968.76 | 8969577 | 63.48 | 607178442 | 117180890 | 40801014 | 157981904 |
| Widows Creek | 1994 | 1965 | 1968.76 | 9447355 | 66.86 | 700312162 | 133722652 | 45616855 | 179339507 |
| Widows Creek | 1995 | 1965 | 1968.76 | 9530981 | 67.45 | 730696957 | 133241583 | 35471319 | 168712902 |
| Widows Creek | 1996 | 1965 | 1968.76 | 8406560 | 58.75 | 765252326 | 116136042 | 53797678 | 169933720 |
| Widows Creek | 1997 | 1965 | 1968.76 | 8002680 | 56.08 | 767011068 | 109606544 | 42829930 | 152436474 |
| Widows Creek | 1998 | 1965 | 1968.76 | 7980620 | 54.88 | 797923680 | 114692109 | 48472996 | 163165105 |
| Cholla | 1988 | 1981 | 1105.44 | 5423223 | 65.26 | 820650041 | 98953532 | 39800254 | 138753786 |
| Cholla | 1989 | 1981 | 1105 | 6910089 | 83.92 | 824668432 | 118244375 | 31238635 | 149483010 |
| Cholla | 1990 | 1981 | 1105 | 5554151 | 67.45 | 828755391 | 94385981 | 23599091 | 117985072 |
| Cholla | 1991 | 1981 | 1105.44 | 5514362 | 67.04 | 720935335 | 92207488 | 25076369 | 117283857 |
| Cholla | 1992 | 1981 | 1105.44 | 6757839 | 81.84 | 723412033 | 110661260 | 30448231 | 141109491 |
| Cholla | 1993 | 1981 | 1105.44 | 6844090 | 80.55 | 834012174 | 107195274 | 28608119 | 135803393 |
| Cholla | 1994 | 1981 | 1105.44 | 6648405 | 78.24 | 837013184 | 111969589 | 29532678 | 141502267 |
| Cholla | 1995 | 1981 | 1105.44 | 5085690 | 58.35 | 838620262 | 89569138 | 26064209 | 115633347 |
| Cholla | 1996 | 1981 | 1105.44 | 5019969 | 57.44 | 843935177 | 88692952 | 31017875 | 119710827 |
| Cholla | 1997 | 1981 | 1105.44 | 6278672 | 72.03 | 840605201 | 100273028 | 26651143 | 126924171 |
| Cholla | 1998 | 1981 | 1105.44 | 6370909 | 73.09 | 842924274 | 90136261 | 29882531 | 120018792 |
| Antelope Valley | 1988 | 1986 | 870 | 6490779 | 82.1 | 0 | 43486447 | 20174147 | 63660594 |
| Antelope Valley | 1989 | 1986 | 870 | 5724763 | 72.61 | 0 | 43488754 | 74556507 | 118045261 |
| Antelope Valley | 1990 | 1986 | 870 | 5717968 | 72.53 | 0 | 44755747 | 82851970 | 127607717 |
| Antelope Valley | 1991 | 1986 | 870 | 5870154 | 74.46 | 0 | 45649212 | 68258814 | 113908026 |
| Antelope Valley | 1992 | 1986 | 870 | 6181667 | 78.19 | 0 | 51093167 | 68850086 | 119943253 |
| Antelope Valley | 1993 | 1986 | 870 | 6540406 | 82.96 | 0 | 52191256 | 67411789 | 119603045 |
| Antelope Valley | 1994 | 1986 | 870 | 6232097 | 79.05 | 0 | 47423198 | 68059621 | 115482819 |
| Antelope Valley | 1995 | 1986 | 870 | 6279201 | 79.64 | 0 | 49237280 | 64017877 | 113255157 |
| Antelope Valley | 1996 | 1986 | 870 | 6250411 | 79.06 | 0 | 51347477 | 66418473 | 117765950 |
| Antelope Valley | 1997 | 1986 | 870 | 6456015 | 81.89 | 0 | 61918691 | 70084130 | 132002821 |
| Antelope Valley | 1998 | 1986 | 870 | 6895069 | 87.46 | 0 | 56408569 | 77363526 | 133772095 |
| Grand River Dam | 1988 | 1986 | 1000 | 6442822 | 72.62 | 769604095 | 61307746 | 11101309 | 72409055 |
| Grand River Dam | 1989 | 1986 | 1000 | 5964425 | 67.41 | 724673395 | 67609454 | 12752596 | 80362050 |
| Grand River Dam | 1990 | 1986 | 1010 | 5475517 | 61.89 | 766545556 | 61030598 | 14343268 | 75373866 |
| Grand River Dam | 1991 | 1986 | 1010 | 6168564 | 69.72 | 766960156 | 63490213 | 16673229 | 80163442 |
| Grand River Dam | 1992 | 1986 | 1010 | 5517697 | 62.19 | 787161838 | 57762531 | 12971184 | 70733715 |
| Grand River Dam | 1993 | 1986 | 1010 | 6216399 | 70.26 | 786850168 | 59970859 | 15837727 | 75808586 |
| Grand River Dam | 1994 | 1986 | 1010 | 6336604 | 71.62 | 786529619 | 66781262 | 13836998 | 80618260 |
| Grand River Dam | 1995 | 1986 | 1010 | 6011664 | 67.95 | 817569727 | 62238185 | 15768101 | 78006286 |
| Grand River Dam | 1996 | 1986 | 1010 | 5995774 | 67.58 | 818783909 | 62807896 | 15001548 | 77809444 |
| Grand River Dam | 1997 | 1981 | 1010 | 6151160 | 69.52 | 789365951 | 63258690 | 16044525 | 79303215 |
| Grand River Dam | 1998 | 1981 | 1010 | 5348523 | 60.45 | 820423124 | 54573694 | 17584318 | 72158012 |
| Plant Name | Year | Year Last Unit Installed | Name Plate Capacity MW | Net Generation MWh | Cap Factor % | Total Cost of Plant $ | Fuel $ | Total Non-Fuel O&M $ | Total Production Costs $ |
| Schahfer | 1988 | 1986 | 1943.46 | 4939319 | 35.34 | 1475695535 | 110363038 | 42699875 | 153062913 |
| Schahfer | 1989 | 1986 | 1943.46 | 6443128 | 46.23 | 1485253817 | 129289774 | 36858594 | 166148368 |
| Schahfer | 1990 | 1986 | 1943.46 | 6382417 | 45.79 | 1508442358 | 126674290 | 47148519 | 173822809 |
| Schahfer | 1991 | 1986 | 1943.46 | 7069049 | 49.66 | 1514641536 | 138822786 | 52325252 | 191148038 |
| Schahfer | 1992 | 1986 | 1943.46 | 6971897 | 48.84 | 1521453234 | 127993051 | 39694045 | 167687096 |
| Schahfer | 1993 | 1986 | 1943.46 | 6736787 | 47.33 | 1529875762 | 122115013 | 41762789 | 163877802 |
| Schahfer | 1994 | 1986 | 1943.46 | 6354041 | 44.64 | 1532566694 | 111134679 | 44870681 | 156005360 |
| Schahfer | 1995 | 1986 | 1943.46 | 7503431 | 52.71 | 1575417394 | 114048060 | 48307171 | 162355231 |
| Schahfer | 1996 | 1986 | 1943.46 | 7724409 | 54.12 | 1600181128 | 118160728 | 43681291 | 161842019 |
| Schahfer | 1997 | 1986 | 1943.46 | 8623828 | 60.58 | 1600236558 | 130223507 | 42644228 | 172867735 |
| Schahfer | 1998 | 1986 | 1943.46 | 9262604 | 65.07 | 1608661102 | 130840374 | 36649244 | 167489618 |
| Welsh (TX) | 1988 | 1982 | 1674 | 7664430 | 55.08 | 438204491 | 176464991 | 8152959 | 184617950 |
| Welsh (TX) | 1989 | 1982 | 1674 | 8479649 | 57.83 | 438401150 | 188134260 | 8565338 | 196699598 |
| Welsh (TX) | 1990 | 1982 | 1674 | 7433288 | 50.69 | 437694422 | 170477278 | 10001594 | 180478872 |
| Welsh (TX) | 1991 | 1982 | 1674 | 8170536 | 58.88 | 438115983 | 186123217 | 11391556 | 197514773 |
| Welsh (TX) | 1992 | 1982 | 1674 | 7887524 | 56.69 | 440314169 | 176324683 | 9485702 | 185810385 |
| Welsh (TX) | 1993 | 1982 | 1674 | 8075481 | 58.2 | 442389175 | 176388414 | 11240180 | 187628594 |
| Welsh (TX) | 1994 | 1982 | 1674 | 7569380 | 54.55 | 446001416 | 152480011 | 10857855 | 163337866 |
| Welsh (TX) | 1995 | 1982 | 1674 | 7441335 | 53.63 | 446987855 | 147103817 | 14197940 | 161301757 |
| Welsh (TX) | 1996 | 1982 | 1674 | 9181009 | 65.98 | 453604741 | 176786786 | 15973984 | 192760770 |
| Welsh (TX) | 1997 | 1982 | 1536.96 | 9884991 | 71.24 | 472536206 | 180117804 | 14121844 | 194239648 |
| Welsh (TX) | 1998 | 1982 | 1535.96 | 10023677 | 72.24 | 476463806 | 169642354 | 17914362 | 187556716 |
| Chesterfield | 1988 | 1969 | 1352.94 | 6760163 | 60.12 | 368797730 | 100342514 | 30718008 | 131060522 |
| Chesterfield | 1989 | 1969 | 1352.94 | 7299739 | 65.1 | 373050113 | 111350003 | 32263720 | 143613723 |
| Chesterfield | 1993 | 1969 | 1352.94 | 7626439 | 68.02 | 387188123 | 111175968 | 30126856 | 141302824 |
| Chesterfield | 1994 | 1969 | 1352.94 | 7218010 | 64.37 | 384756456 | 107101630 | 28779245 | 135880875 |
| Chesterfield | 1995 | 1969 | 1352.94 | 7154327 | 63.8 | 375223902 | 107494231 | 30748042 | 138242273 |
| Chesterfield | 1996 | 1969 | 1352.94 | 7691995 | 68.41 | 380677067 | 114971943 | 16896923 | 131868866 |
| Chesterfield | 1997 | 1969 | 1352.94 | 7483726 | 66.74 | 360140758 | 109745013 | 18707855 | 128452868 |
| Chesterfield | 1998 | 1969 | 1352.94 | 7201560 | 64.23 | 361152443 | 106331151 | 20697673 | 127028824 |

| Plant Name | Year | Year Last Unit Installed | Name Plate Capacity MW | Net Generation MWh | Cap Factor % | Total Cost of Plant, $ | Fuel $ | Total Non-Fuel O&M $ | Total Production Costs $ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chesterfield | 1992 | 1992 | 1828 | 8653597 | 56.39 | 575151681 | 139865586 | 33530470 | 173396056 |
| Chesterfield | 1993 | 1992 | 474.35 | 2359760 | 57.68 | 242307812 | 55721017 | 3420600 | 59141617 |
| Chesterfield | 1994 | 1992 | 474.35 | 2255832 | 55.14 | 244517788 | 52003837 | 5133337 | 57137174 |
| Chesterfield | 1995 | 1992 | 474.35 | 1729866 | 42.29 | 249338696 | 38883406 | 5245714 | 44129120 |
| Chesterfield | 1996 | 1992 | 474.35 | 1034546 | 25.22 | 257004263 | 26862771 | 2757389 | 29620160 |
| Chesterfield | 1997 | 1992 | 474.35 | 937983 | 22.93 | 256189941 | 26255248 | 2086857 | 28342105 |
| Chesterfield | 1998 | 1992 | 474.35 | 1760861 | 43.04 | 252771292 | 44779589 | 8197989 | 52977578 |
| Hermiston Generating Co. | 1996 | 1996 | 468 | 1544880 | 37.26 | 322543580 | 28212508 | 3775956 | 31988464 |
| Hermiston Generating Co. | 1997 | 1996 | 474 | 1923606 | 46.82 | 311837498 | 46835245 | 9025226 | 55860471 |
| Hermiston Generating Co. | 1998 | 1996 | 474 | 3403971 | 82.33 | 308054451 | 67270735 | 9448765 | 76719500 |
| Maalaea | 1993 | 1993 | 154.1 | 594120 | 41.43 | 132779629 | 28268123 | 6837293 | 35105416 |
| Maalaea | 1994 | 1993 | 154.1 | 641624 | 44.66 | 133239710 | 25874991 | 8030766 | 33905757 |
| Maalaea | 1995 | 1993 | 154.1 | 702692 | 49.21 | 133619605 | 29315736 | 8620090 | 37935826 |
| Maalaea | 1996 | 1993 | 174.1 | 739085 | 51.62 | 134735272 | 37500529 | 10010942 | 47511471 |
| Maalaea | 1997 | 1993 | 174.1 | 740350 | 55.24 | 135179866 | 38560785 | 10439301 | 49000086 |
| Maalaea | 1998 | 1993 | 174.1 | 757765 | 54.75 | 193244147 | 30579448 | 10535398 | 41114846 |
| Ocean State Power | 1992 | 1990 | 254.43 | 2187206 | 100 | 207277280 | 52281699 | 8129652 | 60411351 |
| Ocean State Power | 1993 | 1990 | 254.43 | 1859339 | 84.9 | 213897242 | 44506345 | 9423590 | 53929935 |
| Ocean State Power | 1994 | 1990 | 254.43 | 2043779 | 93.32 | 214272264 | 40162292 | 9439989 | 49602281 |
| Ocean State Power | 1995 | 1990 | 254.43 | 1606200 | 73.34 | 165456800 | 40454836 | 10947836 | 51402672 |
| Ocean State Power | 1996 | 1990 | 254.43 | 1933491 | 88.05 | 222814522 | 43947206 | 14698017 | 58645223 |
| Ocean State Power | 1997 | 1990 | 254.43 | 1900440 | 86.78 | 222148309 | 43980694 | 9010255 | 52990949 |
| Ocean State Power | 1998 | 1990 | 254.43 | 2024105 | 92.42 | 222056652 | 46158100 | 8371804 | 54529904 |
| Ocean State Power II | 1992 | 1991 | 254.43 | 2142090 | 100 | 165251985 | 50191388 | 11703734 | 61895122 |
| Ocean State Power II | 1993 | 1991 | 254.43 | 1961710 | 89.58 | 165986717 | 51736527 | 10965202 | 62701729 |
| Ocean State Power II | 1994 | 1991 | 254.43 | 2063409 | 94.22 | 165992883 | 48676319 | 13419612 | 62095931 |
| Ocean State Power II | 1995 | 1991 | 254.43 | 1606356 | 73.35 | 165817092 | 46251952 | 13453740 | 59705692 |
| Ocean State Power II | 1996 | 1991 | 254.43 | 2110773 | 96.12 | 166683948 | 54050720 | 10629477 | 64680197 |
| Ocean State Power II | 1997 | 1991 | 254.43 | 1725412 | 78.79 | 170355191 | 45988796 | 18935874 | 64924670 |
| Ocean State Power II | 1998 | 1991 | 254.43 | 2024105 | 92.42 | 170072934 | 50599194 | 10470202 | 61069396 |
| Richard M. Flynn | 1994 | 1994 | 164 | 717233 | 50.85 | 120725000 | 24148000 | 1250000 | 25398000 |
| Richard M. Flynn | 1995 | 1994 | 164 | 1170210 | 82.97 | 120725000 | 41498000 | 2522000 | 44020000 |
| Richard M. Flynn | 1996 | 1994 | 164 | 885861 | 60.43 | 133332000 | 36096000 | 5927000 | 42023000 |
| Richard M. Flynn | 1997 | 1994 | 164 | 1143973 | 78.22 | 133332000 | 45553000 | 4147000 | 49700000 |
| Richard M. Flynn | 1998 | 1994 | 164 | 1065134 | 72.83 | 133485000 | 47615000 | 5103000 | 52718000 |

| Plant Name | Year | Year Last Unit Installed | Name Plate Capacity MW | Net Generation MWh | Cap Factor % | Total Cost of Plant $ | Fuel $ | Total Non-Fuel O&M $ | Total Production Costs $ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sebring Phillips | 1991 | 1983 | 38.44 | 34055 | 10.23 | 57964878 | 1194503 | 1021051 | 2215554 |
| Sebring Phillips | 1992 | 1983 | 38.44 | 49567 | 15.25 | 59561626 | 1605682 | 1230271 | 2835953 |
| Sebring Phillips | 1993 | 1983 | 38.44 | 40548 | 13.61 | 59741583 | 1301134 | 1056462 | 2357596 |
| Sebring Phillips | 1996 | 1983 | 38.44 | 52916 | 17.72 | 59776347 | 1970669 | 1491952 | 3462621 |
| Sebring Phillips | 1997 | 1983 | 38.43 | 51914 | 17.43 | 59823325 | 1858210 | 964387 | 2822597 |
| Sebring Phillips | 1998 | 1983 | 38.43 | 68328 | 22.94 | 59847501 | 2119704 | 979233 | 3098937 |
| Port Allen | 1995 | 1991 | 43.7 | 207563 | 54.22 | 31975604 | 10983121 | 3716852 | 14699973 |
| Port Allen | 1996 | 1991 | 43.7 | 200448 | 52.22 | 52537351 | 12467455 | 3667825 | 16135280 |
| Port Allen | 1997 | 1991 | 43.7 | 203525 | 53.17 | 57968366 | 13254449 | 3978672 | 17233121 |
| Port Allen | 1998 | 1991 | 43.7 | 233518 | 61 | 31975604 | 15044405 | 3169201 | 18213606 |
| Pennsbury | 1997 | 1996 | 6 | 45165 | 85.93 | 3846648 | 280621 | 307766 | 588387 |
| Pennsbury | 1998 | 1996 | 6 | 15240 | 29 | 3846648 | 95892 | 307766 | 403658 |
| Pebbly Beach | 1995 | 1995 | 9.4 | 25302 | 30.73 | 13767561 | 1727032 | 1248206 | 2975238 |
| Pebbly Beach | 1996 | 1995 | 9.4 | 26690 | 32.32 | 14308358 | 2131975 | 1895848 | 4027823 |
| Pebbly Beach | 1997 | 1995 | 9.4 | 27282 | 33.13 | 14312697 | 2203443 | 1474326 | 3677769 |
| Pebbly Beach | 1998 | 1995 | 9.4 | 28046 | 34.06 | 14312697 | 1716370 | 1523108 | 3239478 |
| Lanai City & Miki Basin | 1995 | 1948 | 10.76 | 27504 | 29.18 | 4410104 | 2331141 | 1100875 | 3432016 |
| Lanai City & Miki Basin | 1996 | 1948 | 15.16 | 28365 | 21.3 | 15877940 | 2650925 | 1169019 | 3819944 |
| Lanai City & Miki Basin | 1997 | 1948 | 15.16 | 27944 | 21.04 | 16372189 | 0 | 0 | 0 |
| Lanai City & Miki Basin | 1998 | 1988 | 12.4 | 27746 | 25.54 | 16818780 | 2003206 | 755424 | 2758630 |
| G.W. Ivey | 1992 | 1981 | 59.11 | 72278 | 14.69 | 38927137 | 2036204 | 2654849 | 4691053 |
| G.W. Ivey | 1997 | 1981 | 59.11 | 47662 | 10.38 | 53218000 | 2143135 | 1816191 | 3959326 |
| G.W. Ivey | 1998 | 1981 | 59.11 | 62520 | 13.62 | 57383407 | 2408086 | 2564512 | 4972598 |

7.0.0	REFERENCES

1.	POWER GENERATION TECHNOLOGIES

Evaluating the Cost of Electricity

By PAUL BREEZE

Published by FINANCIAL TIMES ENERGY

2.	Projected Costs Of Generating Electricity

- Update 1998

NUCLEAR ENERGY AGENCY,

INTERNATIONAL ENERGY AGENCY &

ORGANISATION FOR ECONOMIC

CO-OPERATION AND DEVELOPMENT

3.	Data base of Various Thermal Generation Plants of USA

PROCESSED THROUGH INTERNET

4.	WORLD BANK DISCUSSION PAPERS

Submission and Evaluation of Proposals for

Private Power Generation Projects in Developing Countries

Edited by PETER A CORDUKES

5.	Profile of Power Utilities & Non-Utilities in India – 1997

COUNCIL OF POWER UTILITIES

6.	India’s Energy Sector – September 1996

ECONOMIC INTELLIGENCE SERVICE

CENTRE FOR MONITORING INDIAN ECONOMY (CMIE)

7.	CMIE MONTHLY REVIEW OF INDIAN ECONOMY

8.	CII HANDBOOK OF STATISTICS 1998

9.	RBI BULLETIN

10.	Study of Past Trend in Costs to Evolve a System of

Cost Estimation of Power Projects

by DCL for PFC.