Appendix-IV

Monthly Fuel Price Adjustment (FPA) in Rs/kWh ex-bus for the Energy Charge for Coal/Lignite based generating stations as per following formula:

100

FPA 	= ----------------        (GHRn)    (LPPFm/CVPFm) – (LPPFs/CVPFs)

(100 –AUXn)

– (SFCn)     (CVSFm x LPPFm / CVPFm) – (CVSFs x LPPFs / CVPFs)

Where,

SFCn	–	Normative Specific Fuel Oil consumption in ml/kWh

GHRn   –	Normative Gross Station Heat Rate in kCal/kWh

AUXn	–	Normative Auxiliary Energy Consumption in percentage

LPSFm     –	Weighted Average price of fuel oil on as consumed basis during the month   in Rs./KL.

CVSFm     –	Weighted average GCV of fuel oils fired at boiler front for

the month in Kcal/ml

LPSFs      –	Base value of price of fuel oils as taken for determination of base energy charge in tariff order in Rs. / KL.

CVSFs     –	Base value of gross calorific value of fuel oils as taken for determination of base energy charge in tariff order in Kcal/ml

LPPFm    –	Weighted average price of coal procured and burnt during the  month at the power station in Rs. / MT.

CVPFm    –	Weighted average gross calorific value of coal/lignite fired at boiler front for the month in Kcal/Kg

LPPFs     –	Base value of price of coal as taken for determination of base energy charge in tariff order in Rs. /MT

CVPFs     –	Base value of gross calorific value of coal/lignite as taken for

determination of base energy charge in tariff order in kCal/Kg