Appendix-V

Monthly Fuel price adjustment (FPA) in Rs./kWh Ex-bus for energy charge of  Gas/Naptha based generating stations as per following formula:

100 x   (GHRn) x   (LPPFm/CVPF) – (LPPFs/CVPFs)			FPA   =   	 ---------------------------------------------------------------------

(100 –AUXn)

Where,

FPA    =	Fuel price Adjustment for a month in Paise/kWh Sent out

GHRn   =	Normative Gross Station Heat Rate expressed in kCal/kWh

AUXn	=	Normative Auxiliary Energy Consumption in percentage

LPPFm    =	Weighted average price of Gas or Liquid fuel as per PSL for the month in Rs. / SCM of Rs./ Litre or Rs./Kg

CVPFm    =	Weighted average gross calorific value of Gas or Liquid fuel for the month in Kcal/ SCM or kCal/ Litre or kCal/ Kg

LPPFs     =	Base price of Gas or Liquid fuel as taken for determination of base energy charge in tariff order in Rs. / SCM of Rs./ Litre or Rs./Kg

CVPFs     =	Base value of gross calorific value of Gas or Liquid fuel as taken determination of base energy charge in tariff order in Kcal/ SCM or kCal/ Litre or kCal/ Kg

FPA shall further be subjected to adjustment for monthly operating pattern adjustment (MOPA) for percentage open cycle operation as certified by REB/SLDC and corresponding to Gross Station Heat Rate and auxiliary energy consumption for open cycle operation, as per formula given below:

{(GHRno)/(100-AUXno)}

MOPA = (BEC +FPA) x				      - 1  x  POCM/100

{(GHRnc)/(100-AUXnc)}

Where,

MOPA 	-	Monthly Operating Pattern Adjustment in Rupee/kWh Sent

Out

BEC 	-	Base Energy Charge as per tariff order in Rupee /kWh sent out

FPA 	-	Fuel price Adjustment for the month in Rupee /kWh Sent out

GHRno	-	Normative Gross Station Heat Rate for Open cycle operation expressed in kCal/kWh

GHRnc	-	Normative Gross Station Heat Rate for Combined cycle operation expressed in kCal/kWh

AUXno 	-	Normative Auxiliary Energy Consumption for Open cycle operation in percentage

AUXnc	-	Normative Auxiliary Energy Consumption for Combined cycle operation in percentage

POCM	-	Open cycle generation during the month in percentage